NET511 – Networks 1A Practical Assessment – Week 3 VLANs and Ethernet Virtualization 20 – 26 March 2025 ¦ 10 Marks

| Student Name: _Darium______________________________ | Student Number: 20260425______________________ |
| --- | --- |
| Date Submitted: _______________________________ |  |

# Learning Outcomes

By the end of this practical, you should be able to:

- Explain the purpose of VLANs and how they segment a network at Layer 2

- Create and name VLANs on a Cisco Catalyst switch

- Assign switchports to VLANs using access mode configuration

- Configure and verify 802.1Q trunk links between switches

- Test and interpret intra-VLAN connectivity using ping

- Identify why inter-VLAN routing requires a Layer 3 device

# Instructions

| TIME | You have 2 hours to complete this practical. Manage your time carefully. |
| --- | --- |

| TOOL | Use Cisco Packet Tracer (version 8.x or later) or GNS3. Save your .pkt file. |
| --- | --- |

| NOTE | All evidence must reflect your own work. Screenshots must show your hostname (configured per Task 1). |
| --- | --- |

Read each task fully before you begin configuring. You are expected to consult the relevant RFC documentation, Cisco IOS command references, and your lecture notes where needed. The practical does not walk you through every command — discovering and applying correct syntax is part of the assessment.

# Scenario

You are a junior network engineer at Zola Technologies. The company has a small office with two managed switches (SW1 and SW2) and six PCs. The network manager has asked you to segment the network into three VLANs to isolate traffic between departments. The two switches are connected by an uplink that must carry traffic for all VLANs simultaneously.

Your tasks are to plan, configure, and verify this segmented network using industry-standard IEEE 802.1Q trunking.

# Network Topology

Build the following topology in Packet Tracer. Use 2960 series switches and generic PCs.

TOPOLOGY DIAGRAM PC1 (V10) PC3 (V20) PC5 (V30) ¦ ¦ ¦ Fa0/1 Fa0/3 Fa0/5 +----------SW1----------+ ¦ Gi0/1 (Trunk) ¦ Gi0/1 (Trunk) +----------SW2----------+ Fa0/1 Fa0/3 Fa0/5 ¦ ¦ ¦ PC2 (V10) PC4 (V20) PC6 (V30)

## IP Address Plan

Assign IP addresses according to the following plan. All hosts use the respective switch SVI as their default gateway.

| Device | Interface | VLAN | IP Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| PC1 | NIC | 10 – Management | 192.168.10.10 | /24 |
| PC2 | NIC | 10 – Management | 192.168.10.11 | /24 |
| PC3 | NIC | 20 – Sales | 192.168.20.10 | /24 |
| PC4 | NIC | 20 – Sales | 192.168.20.11 | /24 |
| PC5 | NIC | 30 – IT Support | 192.168.30.10 | /24 |
| PC6 | NIC | 30 – IT Support | 192.168.30.11 | /24 |
| SW1 | VLAN 10 SVI | 10 – Management | 192.168.10.1 | /24 |
| SW2 | VLAN 10 SVI | 10 – Management | 192.168.10.2 | /24 |

## VLAN Plan

| VLAN ID | Name | Assigned Ports (each switch) |
| --- | --- | --- |
| 10 | Management | Fa0/1 – Fa0/2 |
| 20 | Sales | Fa0/3 – Fa0/4 |
| 30 | IT_Support | Fa0/5 – Fa0/6 |

# Tasks

## Task 1: Switch Identification and VLAN Database (2 Marks)

| GOAL | Configure both switches with meaningful hostnames and create the full VLAN database. |
| --- | --- |

Configure SW1 and SW2 with appropriate hostnames. Then, on both switches, create and name all three VLANs as per the VLAN plan above.

Before you configure, consider the following:

- What is the difference between VLAN database mode and global configuration mode for VLAN creation on IOS?

- Where is the VLAN database stored on a Cisco switch, and what file would you expect to see?

- What does the 'active' status mean in the VLAN table output?

Evidence to capture:

- A screenshot of show vlan brief on both SW1 and SW2, showing all three VLANs with correct names and active status.

## Task 2: Access Port Configuration (2 Marks)

| GOAL | Assign each host-facing switchport to its correct VLAN using access mode. |
| --- | --- |

Assign the ports listed in the VLAN plan to their respective VLANs on both switches. Ensure each port is explicitly set to access mode.

Before you configure, consider:

- Why is it important to explicitly set a port to access mode rather than relying on the default?

- What is the DTP (Dynamic Trunking Protocol) implication of not specifying the mode?

- What is the purpose of the spanning-tree portfast command on access ports?

Evidence to capture:

- A screenshot of show interfaces fa0/1 switchport on SW1 (or equivalent) confirming the administrative mode and access VLAN.

- Repeat for at least one port per VLAN on each switch.

## Task 3: 802.1Q Trunk Link Configuration (2 Marks)

| GOAL | Configure the Gi0/1 uplink between SW1 and SW2 as an 802.1Q trunk carrying all three VLANs. |
| --- | --- |

Configure the Gigabit uplink on both switches as a trunk port. Disable dynamic negotiation on the link and explicitly allow only VLANs 10, 20, and 30.

Research and answer the following in your evidence submission:

- What does the native VLAN do on an 802.1Q trunk, and what is the security implication of leaving it at the default?

- Why would you use allowed vlan to restrict which VLANs cross the trunk rather than permitting all?

- What verification command shows the encapsulation type, native VLAN, and allowed VLANs on a trunk port?

Evidence to capture:

- A screenshot of show interfaces trunk on both SW1 and SW2, confirming encapsulation dot1q, mode on, and all three VLANs listed under 'VLANs allowed and active in management domain'.

## Task 4: Intra-VLAN Connectivity Test (2 Marks)

| GOAL | Verify that hosts within the same VLAN can communicate across the trunk link. Confirm that hosts in different VLANs cannot. |
| --- | --- |

Using the IP addresses from the address plan, test the following ping scenarios and record the results:

| Source | Destination | VLAN Relationship | Expected | Actual |
| --- | --- | --- | --- | --- |
| PC1 (192.168.10.10) | PC2 (192.168.10.11) | Same VLAN (10) | Success | Success |
| PC3 (192.168.20.10) | PC4 (192.168.20.11) | Same VLAN (20) | Success | Success |
| PC5 (192.168.30.10) | PC6 (192.168.30.11) | Same VLAN (30) | Success | Success |
| PC1 (192.168.10.10) | PC3 (192.168.20.10) | Different VLAN | Fail | Fail |
| PC1 (192.168.10.10) | PC5 (192.168.30.10) | Different VLAN | Fail | Fail |

For each failed inter-VLAN ping, briefly explain in your submission why the ping fails and what Layer 3 mechanism would be needed to enable communication.

Evidence to capture:

- Screenshots of all five ping results from the Packet Tracer PDU window or CLI.

## Task 5: Topology Diagram (2 Marks)

| GOAL | Produce a clear, labelled network topology diagram reflecting your completed configuration. |
| --- | --- |

Create a topology diagram (hand-drawn and scanned, or drawn using any diagramming tool) that includes the following:

- All six PCs with IP addresses and VLAN membership labelled

- Both switches with hostnames

- Trunk link clearly marked with encapsulation type and allowed VLANs

- Access ports labelled with their VLAN ID

- Switch SVIs labelled with IP addresses

A diagram that is unlabelled or incomplete will not receive full marks. Neatness and accuracy are both assessed.

# Submission Requirements

| DUE | Submit via the Githubby 23:59 on Sunday 26 March 2025. |
| --- | --- |

Your submission must include the following in a single ZIP archive:

- Your Packet Tracer (.pkt) file, saved with your student number as the filename (e.g., 20241234.pkt).

- A PDF or Word document containing all required screenshots, clearly labelled per task.

- Your completed topology diagram (scanned or exported as a PDF/PNG).

- A brief written reflection (100 – 150 words) addressing: What is the main limitation of a Layer 2-only VLAN network? What would you add to enable full inter-departmental communication?

Screenshots must show your switch hostname in the CLI prompt. Screenshots that do not show the hostname will not be accepted as evidence.

# Academic Integrity

This is an individual assessment. You may discuss concepts with peers, but all configurations, screenshots, and written responses must be your own. Sharing of .pkt files or screenshots is considered academic misconduct and will be referred to the Disciplinary Committee.

# Marking Rubric

| Task | Evidence Required | Marks | Score |
| --- | --- | --- | --- |
| Task 1: VLAN Creation & Naming | show vlan brief output (all 3 VLANs active) | 2 |  |
| Task 2: Access Port Assignment | show interfaces switchport per host port | 2 |  |
| Task 3: 802.1Q Trunk Configuration | show interfaces trunk on SW1 and SW2 | 2 |  |
| Task 4: Intra-VLAN Connectivity Test | Successful pings within each VLAN | 2 |  |
| Task 5: Topology Diagram | Labelled diagram with IPs, VLANs and trunk links | 2 |  |
| TOTAL |  | 10 |  |

Marks will be deducted for missing evidence, incorrect hostnames in screenshots, or configurations that do not match the address/VLAN plan.

# Reference Resources

The following resources are available to you. You are expected to use them — referencing documentation is a professional skill.

- Cisco IOS Command Reference: https://www.cisco.com/c/en/us/support/ios-nx-os-software

- IEEE 802.1Q Standard (overview): https://standards.ieee.org/ieee/802.1Q

- Packet Tracer Download: https://www.netacad.com

- Lecture slides – Week 2 and Week 3 (available on LMS)

| TIP | If you are stuck on a concept, check the Cisco Learning Network forums or your textbook before asking the lecturer. Problem-solving independently is a core networking skill. |
| --- | --- |